SpeedVal Kit Simulation Quick-Start

This is a quick-start guide on getting the SpeedVal Kit Simulation opened and running in LTSpice.

- Double-click to open the example circuit for DPT: P_MOD_MB-HB-900V-40A_DPT.asc

- You’ll note that the simulation is pre-loaded with a card (P_MOD-PWR-MM-C3M0045065K) – to change this card click on the ‘AND’ symbol in LTSpice to select a component (highlighted in yellow below)

- This will open an interface and you’ll need to select the folder containing the simulation environment at the top – folder name is SpeedVal Kit Simulation (see below)

- You’ll see all of the available Daughter Cards and Gate Driver cards. Simply delete the existing card in the simulation and replace it with the card of your choice.

SpeedVal Kit Simulation User Guide

This section details how to make measurements in in the SpeedVal Kit simulation for the high and low-side devices.

High-side Device

The high-side device has a measurement array built-in to the Daughter Card model (highlighted below):

- Idsh – current passing through the device

- Vdsh – voltage measured from drain to source

- Vgsh – voltage measured from gate to source

- Tj_h – junction temperature of the device

- Tc_h – case temperature of the device

Note that the temperature measurement nodes can be fixed by connecting an external voltage source to either pin. For example, connecting to the Tj_h node will fix the device junction temperature.

Low-side Device

The low-side device mimics the structure of the high-side device, having access to the same five nodes for measurement (or setting with respect to the Tj/Tc).

- Ids – current passing through the device

- Vds – voltage from drain to source (note the existence of a node, Vdrain)

- Vgs – voltage from gate to source

- Tj_l – junction temperature of the device

- Tc_l – case temperature of the device

Measuring Quantities

The temperature and voltage measurements can be taken by using the simple voltage probe available in LTSpice after running the simulation.

Voltage Measurement:

Below is an example showing how to probe the device drain-source voltage.

Current Measurement:

Below is an example demonstrating how to probe the device current.

Waveforms:

Below is the waveform window showing the aforementioned example measurements.

DPT-Specific Measurement

This simulation does provide functionality to analyze the switching events using a simulation file called “P_MOD_MB-HB-900V-40A_SingleDPT.asc” where a measurement process exists to provide the following information about the switching events:

- Eon and Eoff vs. Current

- didt and dvdt vs. Current (on and off)

Note that these pre-defined MEASURE statements in LTSpice are designed around the provided set of parameters. Variations in pulse width, duty cycle, or excessive ringing in an application may result in the statement failing to process or yielding inaccurate results.

To view these values after running the simulation, click ‘View’ -> ‘SPICE Error Log’

The SPICE Error Log will open. Right-click inside the window and select ‘Plot step’d .meas data’

An empty plot window will open, with the x-axis being the swept current levels for the simulation. To add waveforms, right-click in this window and click ‘Add Traces’. In the new pop-up, select the traces you want to see.

This selection will plot the turn-on energy (Eon) over current in the window as shown below:

NOTE: For troubleshooting purposes, sometimes replacing a Power Daughter Card model with another will change the reference for the measurement via the instance name. A quick check to ensure the new model card will work with the pre-defined measure statement is to right-click on the Power Daughter Card model and make sure the instance name is ‘X2’ (below).

If it is not, change this value to ‘X2’. Be sure to check the Gate Driver Card model to ensure that it does NOT have the same instance name as the Power Daughter Card model.